_________________________________

VideoToolkit™ Family Specifications

Preliminary Specifications - 2/28/95

Confidential

©1995 Abbate Video, Inc.

CONTENTS

WELCOME

ABOUT AV/net

AV/net Q/A

FEATURES/BENEFITS

SPECIFICATIONS

CONTACTING ABBATE

WELCOME

Abbate Video is pleased to announce the AV/net family of networked intelligent device control modules.  The AV/net family  offers the widest range of affordable and powerful device control options available. The AV/net family controls VCRs, Camcorders, GPIs and other devices directly from your computer's serial port on a daisy chainable network.

ABOUT AV/net

An AV/net™ module (node) is a microcontroller based device controller. The AV/net™ software drivers let programs communicate with the AV/net modules, and in turn control devices connected to those modules.

Software packages such as VideoToolkit™, the VTK Plug In Packs for Premiere, VTK for Newton, VTK for PenEase, and others use AV/net™ software drivers to control devices connected to the AV/net modules.

An AV/net equipped package includes:

A host computer to AV/net connection cable.

An AV/net module for a particular type of device.

Software that knows how to communicate with the AV/net network.

Additional AV/net modules (nodes) are available.  Additional modules may control VCRs, trigger GPI devices (General Purpose Interface contact closures), read timecode, and more.

AV/net Q/A

What is it?

How does it work?

How smart is it?

What makes it better?

What software packages are AV/net aware?

All products in the VideoToolkit Family are AV/net aware.  These include:

VideoToolkit

VTK Plug In Pack for Adobe Premiere (Mac)

VTK Plug In Pack for Adobe Premiere (Windows)

Third party products include:

FormBase from Software Technologies

PenEase for the Newton from Software Technologies

MacShapa

Does Abbate Video OEM AV/net modules and drivers?

Yes.  OEM arrangements can be structured in different ways. Low level AV/net code can be licensed. High level AV/net code can be licensed. AV/net modules can be bundled. Also, the AV/net microcontroller can be licensed and added to a card.  The AV/net connection on that card can function as an AV/net connection to the outside world.  The end user can then add AV/net modules as needed.

FEATURES/BENEFITS

The AV/net architecture is:

Daisy chainable

A single serial port on the host computer is all that is required to connect 7 devices.

Platform Independent Hardware

An AV/net module can be connected to Macintosh RS422 serial ports, PC RS232 serial 		ports or PDA serial ports.

Modular and Upgradable

Future nodes will provide GPI and timecode capabilities.

Low cost and modular

The software and hardware design is modular and well suited for on-board and 		add-on OEM applications.

Standard serial interface

No custom low level drivers needed on host computer.

SPECIFICATIONS

Serial Ports Supported:

Host computer with an available RS232/422 port. For example

Macintosh RS422 serial ports (modem or printer).

IBM PC compatible COM ports 1-4 located at the standard base address.

Newton RS422 serial port.

Control Protocols Supported:

Source camcorder or VCR or other device supporting one of the following remote control protocols:

Control-L (LANC)

Control-S

Infrared

Control-M (Panasonic 5-pin)

ViSCA

RS-422 Sony compatible protocol

RS-232

GPI

Network Connections:

Host to AV/net connection:

Mac / PDA	-	8 pin mini-din to RJ11

PC RS232	-	modular 9 pin F D  and 6p/6c RJ11 to RJ11 (reverse wiring)

6'

AV/net node to node connection:

6p/6c RJ11 to RJ11 (reverse wiring)

3'

AV/net node to controlled device connection:

An AV/net node is programmed and wired to control a particular type of device.

Control-L (LANC)	-	2.5mm Stereo Sub-mini

Control-S		-	3.5mm mono mini-plug

Infrared	-	IR sender module attached to Control-S mini-plug

Control-M		-	5 pin Panasonic style connector

ViSCA			-	8 pin M mini-din

RS-422		-	9 pin M D

RS-232		-	assorted

GPI			-	RJ11 to Terminal Block termination

Power Requirements:

AV/net nodes that are connected to powered devices such as camcorders and vcrs derive power from those devices.

AV/net nodes that are connected to un-powered devices such as GPI triggers are powered from the host computer connection, or from another AV/net node that is connected to a powered device.

System Requirements:

Macintosh:

Mac Plus or greater

IBM PC Compatible:

Microsoft Windows 3.1 or greater

Available 9 pin Serial Port, COM 1 through 4, located at standard base address

Newton:

All

Controllable Camcorder or VCR (see list)

AV/net Part Numbers:

AVN-805	Sony LANC

SOURCE:	LANC (Control-L) camcorder or VCR

AVN-806	ViSCA

SOURCE:	Visca Vbox or VCR

AVN-822	RS-422

SOURCE:	RS-422 BVU/BVW/PVW/UVW1600-1800

EVO/VO-9800/9850/SVO-95xx

AG7650-7750-850/BRS622-822/EditDesk

AVN-810	Panasonic Control-M

SOURCE:	AG-1830/1960/1970 FS-88/100/200/1000

AVN-811	Panasonic RS-232

SOURCE:	AG-5700/TQ3031

AVN-812	Mitsubishi RS-232

SOURCE:	BV2000

AVN-814	Sanyo RS-232

SOURCE:	GVRS950

Accuracy:

Typical accuracy when used with RC timecode is +/- 1 frames. Typical accuracy when used with LTC/VITC timecode is +/- 0 frames. Otherwise, accuracy depends on tape counter/transport accuracy of controlled device.

VCR/Camcorder Configuration:

AV/net nodes controls camcorders and VCRs using simple control cables. The AV/net cable plugs into your computer's serial port (PC: COM 1 through 4, Mac:  Modem/Printer/networked serial device). The other end plugs into the AV/net node RJ11 connector. The AV/net node cable plugs into the remote control port on your source camcorder or VCR.

Sources must have remote control ports capable of two-way communication (like a data bus). This is because VideoToolkit must be able to both send commands to the camcorder or VCR, as well as receive status information back from it (like time code or tape counter location data).

Examples of two-way control ports include Control-L (LANC) (found on most Hi-8 and 8 mm camcorders and VCRs, and some Sony S-VHS and VHS VCRs), Control-M Panasonic 5-pin (found on the AG1830/1960/1970 vcrs), ViSCA (found on the Sony Vdeck and Vbox), RS-422 (found on professional decks), and RS-232 (found on professional and "prosumer" decks).

In all cases, you will control your source by connecting a control cable to its two-way control port.

License:

Non-exclusive license is for single-location.

Custom Site-License packages available.

Custom OEM License packages available.

Compatible Video Equipment:

This is only a partial list of supported equipment.  Since new devices arrive frequently new support is added to VideoToolkit regularly. If a device you'd like to control is not listed check with Abbate Video for the most current list.

Canon

Al                		Hi-8 Camcorder

Al/Digital   		Hi-8 Camcorder

Al/MKII      		Hi-8 Camcorder

LI PRO        		Hi-8 Camcorder

L2 PRO       		Hi-8 Camcorder w/RC

Nikon

VN-500       		8mm Camcorder

VN-950/960   	Hi-8 Camcorder

Ricoh

R-66            		8mm Camcorder

R-86S          		8mm Camcorder

R-67            		8mm Camcorder

R-88          	 	Hi-8 Camcorder

R-105S         	8mm Camcorder

R-108H         	Hi-8 Camcorder w/RC

R-800           	Hi-8 Camcorder

R-830/831   	8mm Camcorder

R-861          		8mm Camcorder

Sony VCRs

EVA-50           	8mm Player/Recorder

EVC-3                         8mm Player/Recorder

EVC-8U (*)    	8mm Player/Recorder

EVC-40  		8mm Player/Recorder

EVC-100      		Hi-8 Player/Recorder

EVA-50          	8mm Player/Recorder

EVS-550       		8mm Player/Recorder

EV-DT2             	8mm Player/Recorder/Monitor

EVO-9100        	Hi-8 Camcorder w/Hi-8TC

EVS-550/800     	8mm Player/Recorder

EVS-900/3000  	Hi-8 Player/Recorder

EVS-1000		PAL Hi-8 Player/Recorder

EVS-7000		Hi-8 Player/Recorder w/RC

EVS-9000		PAL Hi-8 Player/Recorder w/RC

GV-500               	Walkman

GVS-50                 	Walkman

SLV-30/50/70     	VHS Player/Recorder

SLV-676/757       	VHS Player/Recorder

SVO-160                	VHS Player/Recorder

SLV-R5             	S-VHS Player/Recorder

SLV-1000          	S-VHS Player/Recorder

Visca

CI-1000              	V-Box Computer Interface

CVD-1000		Hi-8 Player/Recorder w/RC

CVD-500		8mm Player/Recorder w/RC

Betamax

Assorted L equipped models

Sony Camcorders

CCD-F33/35		8mm Camcorders

CCD-F40/50/55     	8mm Camcorders

CCD-70              	8mm Camcorders

CCD-V9/11    	8mm Camcorders

CCD-V220      	8mm Camcorder

CCD-V201/301	8mm Camcorders

CCD-V401/501  	8mm Camcorders

CCD-V99/101/701 	Hi-8 Camcorders

CCD-V801       	Hi-8 Camcorder  w/RC

CCD-V5000       	Hi-8 Camcorder

CCD-V6000   	PAL Hi-8 Camcorder w/RC

CCD-SP9      		8mm Camcorder

FX-310/410/510	8mm Camcorder

FX-710         		Hi-8 Camcorder

CCD-TR4/5/6/7 	8mm Camcorder

CCD-TR51              	8mm Camcorder

CCD-TR71              	8mm Camcorder

TR81/101/200       	Hi-8 Camcorder

TR700         	 	Hi-8 Camcorder w/RC

CCD-VX3                   	Hi-8 Camcorder w/RC

Yashica

KDH-170        	8mm Camcorder

KDM-770        	8mm Camcorder

KX-70/90       	8mm Camcorder

KX-77          		8mm Camcorder

KD-H1          		Hi-8 Camcorder

KDM-170        	Hi-8 Camcorder w/RC Time Code

Panasonic Control-M

AG-1950/1830/1960/1970	S-VHS

FS-88/100/200			PAL S-VHS

FS-1000				PAL S-VHS w/VITC option

RS422 Protocol

Sony

BVU 8xx/9xx		U-Matic

VO-9800/9850	U-Matic

BVW			Betacam

PVW			Betacam

UVW 1600/1800	Betacam

EVO-9800/9850	Hi8

Panasonic

AG-7650/7750	S-VHS Feeder/Editor

AG-850		S-VHS Editor

JVC

BR-S822U      	S-VHS Editor

BR-S622U      	S-VHS Source

Edit Desk Family	S-VHS

RS232 Protocol

Mitsubishi

BV-2000 		S-VHS Player/Recorder

Panasonic

AG-5700        	S-VHS Player/Recorder

TQ-3031		Writeable Optical

Sanyo

GVR-S950        	S-VHS Player/Recorder w/LTC

CONTACTING ABBATE

Abbate Video, Inc.

14 Ross Ave, Floor 3

Millis, MA 02054

Telephone (508) 376-3712   Fax (508) 376-3714

InterNet: mpa@abbate.com or ftp.abbate.com

AppleLink: "Abbate.Video"

AOL: "Abbate" or "Abbate Vid"

Online Support: America OnLine: Keyword   "Abbate"